## EE 330L Energy Systems Lab (Fall 2026)

## Laboratory 4  Three-Phase Loads- Part 1

Introduction

In this laboratory, the voltages, currents, impedances, phase angles, power factors, and real & reactive power in three-phase systems will be measured and studied.  In particular, a balanced 4-wire Y-Y, unbalanced 4-wire Y-Y, balanced 3-wire Y-Y,  unbalanced 3-wire Y-Y, and balanced  three-phase systems will be considered.  In addition to using three power meters, the two power meter method is introduced.

# Preliminary

A.  Balanced 4-wire Y-Y Three-Phase System

Figure 1  Balanced 4-wire Y-Y Three-Phase System

As shown in Figure 1, we will examine a balanced Y-Y three-phase system with a physical neutral (N) wire present.  For the circuit, the phase voltage is Vf = 69.3 Vrms and , , & .  The Y-configured loads are .

- Compute the RMS phasor currents as well as RMS phasor line-to-line voltages , , &  shown in Figure 1.  Then, complete Table 1 with RMS magnitudes (not phasors).

Table 1  Currents and Voltages for Balanced 4-wire Y-Y Three-Phase System

| Quantity | Value |
| --- | --- |
| IL = Ia = Ib = Ic |  |
| In |  |
| VLL = Vab = Vbc = Vca |  |

- Compute the ratio VLL / Vf.  Is the ratio consistent with the text (give text equation)?

- Next, use the phasor RMS currents and voltages to compute the complex powers for the three phases as well as overall.  Then, complete Table 2 with real, reactive, & apparent  powers and power factors (PFs).

Table 2  Powers and PFs for Balanced 4-wire Y-Y Three-Phase System

| Phase/Total | P (W) | Q (var) | S (VA) | PF |
| --- | --- | --- | --- | --- |
| a |  |  |  |  |
| b |  |  |  |  |
| c |  |  |  |  |
| total |  |  |  |  |

B.  Unbalanced 4-wire Y-Y Three-Phase System

Figure 2  Unbalanced 4-wire Y-Y Three-Phase System

Next, we will examine an unbalanced Y-Y three-phase system with a physical neutral (N) wire present as shown in Figure 2.  The phase voltage is Vf = 69.3 Vrms and , , & .  The loads are , , & .

- Compute the RMS phasor currents as well as RMS phasor line-to-line voltages , , &  shown in Figure 2.  Then, complete Table 3 with RMS magnitudes.

Table 3  Currents and Voltages for Unbalanced 4-wire Y-Y Three-Phase System

| Quantity | Value |
| --- | --- |
| Ia |  |
| Ib |  |
| Ic |  |
| In |  |
| VLL = Vab = Vbc = Vca |  |

- Next, use the phasor RMS currents and voltages to compute the complex powers for the three phases as well as overall.  Then, complete Table 4 with real, reactive, & apparent  powers and PFs.

Table 4  Powers and PFs for Unbalanced 4-wire Y-Y Three-Phase System

| Phase/Total | P (W) | Q (var) | S (VA) | PF |
| --- | --- | --- | --- | --- |
| a |  |  |  |  |
| b |  |  |  |  |
| c |  |  |  |  |
| total |  |  |  |  |

C.  Balanced 3-wire Y-Y Three-Phase System

Figure 3  Balanced 3-wire Y-Y Three-Phase System

Next, we will examine a balanced Y-Y three-phase system without a neutral (N) wire (Figure 3).  For the circuit, the phase voltage is Vf = 69.3 Vrms and , , & .  The Y-configured loads are .

- Compute the RMS phasor currents shown in Figure 2 as well as RMS phasor line-to-line voltages , , & .  Then, complete Table 5.  [Hints: You CAN use a per-phase equivalent circuit as the voltages at n and n’ are the same.  Also, KVL/mesh analysis works and  KCL is always true.]

Table 5  Currents and Voltages for Balanced 3-wire Y-Y Three-Phase System

| Quantity | Value |
| --- | --- |
| IL = Ia = Ib = Ic |  |
| VLL = Vab = Vbc = Vca |  |

- Next, use the phasor RMS currents and voltages to compute the complex powers for the three phases as well as overall.  Then, complete Table 6 with real, reactive, & apparent  powers and PFs.

Table 6  Powers and PFs for Balanced 3-wire Y-Y Three-Phase System

| Phase/Total | P (W) | Q (var) | S (VA) | PF |
| --- | --- | --- | --- | --- |
| a |  |  |  |  |
| b |  |  |  |  |
| c |  |  |  |  |
| total |  |  |  |  |

D.  Unbalanced 3-wire Y-Y Three-Phase System

Figure 4  Unbalanced 3-wire Y-Y Three-Phase System

Next, we will examine an unbalanced Y-Y three-phase system without a neutral (N) wire (Figure 4).  The phase voltage is Vf = 69.3 Vrms and , , & .  The Y-configured loads are , , & .

- Compute the RMS phasor currents as well as RMS phasor line-to-line voltages , , &  shown in Figure 4.  Then, complete Table 7 with RMS magnitudes.  [Hints: You can NOT use a per-phase equivalent circuit as the voltages at n and n’ are different, i.e.,  is NOT across  etcetera.  KVL/mesh analysis works.  KCL is always true.]

Table 7  RMS Currents and Voltages for Unbalanced 3-wire Y-Y Three-Phase System

| Quantity | Value |
| --- | --- |
| Ia |  |
| Ib |  |
| Ic |  |
| VLL = Vab = Vbc = Vca |  |

- Next, use the phasor RMS currents and source phase voltages to compute the complex powers (e.g., ) for the three voltage source phases as well as the overall source.  Then, complete Table 8 with real, reactive, & apparent  powers and PFs.

Table 8  Source Powers and PFs for Unbalanced 3-wire Y-Y Three-Phase System

| Phase/Total | P (W) | Q (var) | S (VA) | PF |
| --- | --- | --- | --- | --- |
| a |  |  |  |  |
| b |  |  |  |  |
| c |  |  |  |  |
| total |  |  |  |  |

- Next, use the phasor RMS currents and load impedances to compute the complex powers (e.g., ) for the three loads as well as the overall load.  Then, complete Table 9 with real, reactive, & apparent  powers and PFs.

Table 9  Load Powers and PFs for Unbalanced 3-wire Y-Y Three-Phase System

| Phase/Total | P (W) | Q (var) | S (VA) | PF |
| --- | --- | --- | --- | --- |
| a |  |  |  |  |
| b |  |  |  |  |
| c |  |  |  |  |
| total |  |  |  |  |

E.  Balanced D-D Three-Phase System

Figure 5  Balanced D-D Three-Phase System

Next, we will examine a balanced D-D three-phase system as shown in Figure 5.  The phase voltage is Vf = 120 Vrms and , , & .  The D-configured loads are .

- Compute the RMS phasor currents shown in Figure 5.  Then, complete Table 10 with RMS magnitudes.

Table 10  Currents and Voltages for Balanced D-D Three-Phase System

| Quantity | Value |
| --- | --- |
| If = Iab = Ibc = Ica |  |
| IL = Ia = Ib = Ic |  |
| VLL = Vab = Vbc = Vca |  |

- Compute the ratio IL / If.  Is the ratio consistent with the text (give text equation)?

- Next, use the phasor RMS currents and voltages to compute the complex powers (e.g., ) for the three load phases as well as the overall load.  Then, complete Table 11 with real, reactive, & apparent  powers and power factors (PFs).

Table 11  Load Powers and PFs for Balanced D-D Three-Phase System

| Phase/Total | P (W) | Q (var) | S (VA) | PF |
| --- | --- | --- | --- | --- |
| ab |  |  |  |  |
| bc |  |  |  |  |
| ca |  |  |  |  |
| total |  |  |  |  |

F.  Two Power Meter (AKA Two-Wattmeter) Method

For a real-world D-configured load, we often would NOT be able to measure the phase currents , , & ; all we would have access to is three external load terminals.  Similarly, we would not have access to node n’ in a Y-configured load with a 3-wire connection.  In these cases, we can employ the two power meter (AKA two-wattmeter) method which uses two line-to-line voltages and two line currents, as shown in Figure 8, to find the overall powers to the load (or from the source).

Figure 8  Two Power Meter (AKA Two-Wattmeter) Method

Since this method relies only on the line quantities, the load and source configurations do not matter (any combination of D and/or Y).  Further, this measurement method works with balanced and unbalanced 3-wire systems.  It can be shown that:

.

The two power meter (AKA two-wattmeter) method only works on balanced 4-wire systems (proof depends on  which is true only when ).  This method does not work on unbalanced 4-wire systems where .

- For each of the preceding three-phase systems, use previously calculated values of Ptot, Qtot, Stot, & PF = cosqtot and complete Table 12.

Table 12  Total Powers & PFs for Three-Phase Systems

| System | Ptot (W) | Qtot (var) | Stot (VA) | PFtot |
| --- | --- | --- | --- | --- |
| A. Balanced 4-wire Y-Y |  |  |  |  |
| B. Unbalanced 4-wire Y-Y |  |  |  |  |
| C. Balanced 3-wire Y-Y |  |  |  |  |
| D. Unbalanced 3-wire Y-Y |  |  |  |  |
| E. Balanced D-D |  |  |  |  |

- For each of the preceding three-phase systems, determine or use previously calculated values of , , , &  to compute  using the two power meter (AKA two-wattmeter) method.  Then, find Ptot, Qtot, Stot, & PF = cosqtot and complete Table 13.

Table 13  Total Powers & PFs for Three-Phase Systems using Two Power Meter Method

| Three-Phase System | Ptot (W) | Qtot (var) | Stot (VA) | PFtot |
| --- | --- | --- | --- | --- |
| A. Balanced 4-wire Y-Y |  |  |  |  |
| B. Unbalanced 4-wire Y-Y |  |  |  |  |
| C. Balanced 3-wire Y-Y |  |  |  |  |
| D. Unbalanced 3-wire Y-Y |  |  |  |  |
| E. Balanced D-D |  |  |  |  |

- Compare Table 12 with Table 13.  For which three-phase system(s) do the results agree?  For which three-phase system(s) do the results disagree?

Each group should write and submit one hard copy of a lab report on Part 1 of Lab 4 to the instructor following guidelines given in lab syllabus.

Due Friday October 2, 2026 by 4 pm to instructor’s office or EECS mailbox.